# SSH Proxy Script README

## Overview

This script provides a secure and flexible way to establish SSH sessions through a proxy using SSH keys. It supports interactive and non-interactive modes, remote command execution, and detailed logging.

## Capabilities

- Interactive or CLI-based SSH session through a proxy using SSH keys.

- Separate usernames for proxy and remote hosts.

- Single remote command execution using --remote-cmd.

- Multiple commands from a file using --cmd-file.

- Stop-on-error option (--stop-on-error) for command files.

- Daily logging of all events with timestamps in the user's directory.

- Debug mode (-d) prints all events to stdout.

- Help summary (-h) with usage and examples.

## Usage

1 ssh_proxy.sh [options]

### Options

- --user <username>: Specify SSH username for both proxy and remote.

- --proxy-user <username>: Specify SSH username for proxy only.

- --remote-user <username>: Specify SSH username for remote only.

- --proxy <proxy_host>: Specify proxy host.

- --remote <remote_host>: Specify remote host.

- --remote-cmd "<command>": Run a single command on the remote host.

- --cmd-file <path>: Run multiple commands from a file on the remote host.

- --stop-on-error: Stop executing commands from file on first failure.

- -d: Enable debug mode (prints events to stdout).

- -h: Show help message.

### Examples

Interactive session:

1 ./ssh_proxy.sh

Single command:

1 ./ssh_proxy.sh --proxy-user bob --remote-user alice --proxy proxy1.example.com --remote remote2.example.com --remote-cmd "uptime" -d 2

Multiple commands from file:

1 ./ssh_proxy.sh --proxy-user bob --remote-user alice --proxy proxy1.example.com --remote remote2.example.com --cmd-file /tmp/commands.txt --stop-on-error -d

### Example commands.txt Template

1 # Each line represents a command to run on the remote host 2 uptime 3 ls -l /var/log 4 df -h 5 cat /etc/os-release 6

Commands are executed sequentially for safety. If --stop-on-error is used, execution halts on the first failure.

## Logging

- Logs are stored in the user's directory under:

1 ssh_proxy_connection_log_<MM-DD-YY>.log

- Each entry includes timestamp, proxy/remote user, hosts, mode, and status.

## Security Notes

- SSH key-based authentication only (no password SSH).

- Password prompt is for local sudo use only.

- Ensure log files have restricted permissions (chmod 600).

## Requirements

- Bash (macOS or Linux)

- SSH client

- Valid SSH key

## License

This script is provided as-is for secure SSH proxy operations.